1.                HARD DISK PREPARATION program 3.33

KFH ASSOCIATES,           1985

2771, Middleborough

SAN JOSE, CA 95132

FOR IBM PC   AND  XT & COMPATIBLES

AT needs HDPREPAT.exe

DOS 2.0/2.1/3.0/3.1

ALSO  XEBEC 1210A

DTC   5150BX

ADAPTEC 2002A

W DIGITAL 1002

ETC. PC CONTROLLER BOARDS

** If you think the program is useful  to you,

$35.00 donation to    H. King   at above

address will register for updating service

a. 3.3 reduce the program size and enhcanced the option 3 option

b. Enhanced the track entering and display feature

c. Added the scan track feature to locate the already marked track

in option 7 and 8.

2.  The program READ in the hard disk drive parameters from the

controller to determine the available number of heads and cylinders

for each attached drive, if the controller follows the IBM BIO

convention. Otherwise, the program defaults to 2 head and 100

cylinders for each drive. When this happens, the controller usually

has problem. You can use option 9 to change this settings, if you

have an Adaptec controller.

3.  Hard disk preparation uses the controller default parameters in the

BIOS.

Instructions:

The program is manual driven and self explanatory. The following is a

summary.

Options:

1. Select the drive to be operated on. Drive 0 is the first hard disk

drive attached to the controller (usually the C:), Drive 1 is the

second one (usually the D:).

2. Perform the controller FORMAT of the specified drive by option 1.

See option 3. Run FDISK and Dos FORMAT after this step.

Bad track locations can be input at this step and the program

will use the input to mark the bad ones accordingly.

3. Perform the controller FORMAT of the specified drive by option 1

and scan the disk for bad tracks and mark them bad. Run FDISK and

DOS FORMAT after this step. This option 3 is preferred to option 2.

In the begining of this step, you can enter the bad tracks if you

already know them.  This step marks  any bad track found,

the ones you entered,  at the end of the process.

4. Move the head to track 0 for the specified drive.

5. Exit this program.

6. Set the interleave number for the controller FORMAT. The default is

6 which is used by IBM PC/XT. Do not use other number unless you

have enough knowedge on the controller. Adaptec controller can use

interleave 2.

7. If you know a specific track and head location is bad, you can use

this option to mark the track bad. Use DOS FDISK AND FORMAT after

this step.

Option allows the program to scan the drive and lists tracks that

already marked bad.

8. If you want to remark a bad track to reuse the track, you can use this

option. This option can not physically correct the defect track.

Option allows the program to scan the drive and lists tracks that

already marked bad.

9. Use this option to define the number of heads and cylinders for each

drive for this program to use. This parameter does  overwrite

the controller defined parameter and affect the controller FORMAT

operation. The BIOS parameters are not affected. Warning: The DOS

FORMAT uses BIOS parameters.

However, the ADAPTEC 2002A is an exception, the overwrite parameters

are recognized by DOS Fdisk and Format programs.

A. The same operation as the option 3, except during the verify process

more data patterns are used. The process can take twice longer then

the option 3. Usually, the option 3 is good enough.

Note:

At option 2,3,7 and 8, head and track location input feature allows

the number entering easier and can be edited later. Once the total track

location display is on, you can use the cursor key to move the

pointer (reversed video) to different line/column and use command

to change (C),  remove (R), Re-enter every thing (N), go ahead to

proceed (Y). The editing options are displayed at the bottom of the

screen for reference.

If a new item need to be added, use cursor key to move the pointer

to a location that does not have any entry, hit "C" - change, head

number and cylinder number can then be added.

################################################################